| Course Name: | Hardware Description Language Lab (2UXL401) | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: | 04 / 05 / 2021 | Batch No: | B2 |
| Faculty Name: | Prof. Bhargavi Kaslikar | Roll No: | 1912060 |
| Faculty Sign & Date: |  | Grade/Marks: |  |

Experiment No: 8

Title: Use of Functions and Procedures

Aim and Objective of the Experiment:

Write a code for Universal Shift Register with operations as 00: Shift Left , 01: Shift Right, 10: Rotate Right, 11: Load . Write functions for all operations and use them. Test using test bench and Implement the same on CPLD Kit. To study functions and Procedures in VHDL

COs to be achieved:

CO 2: Test a VHDL code and verify the circuit model. CO 3: Synthesize and Implement the designed circuits on CPLD/ FPGA.

Work to be done

Upload VHDL codes. Also upload test bench and simulation for the same.

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_arith.all; use ieee.std_logic_unsigned.all; entity barrels is port( din: in std_logic_vector(7 downto 0); shift: in std_logic_vector(2 downto 0); con: in std_logic_vector(1 downto 0); dout: out std_logic_vector(7 downto 0) ); end barrels; architecture barrels_arch of barrels is --function to convert shift input to integer function conv_int(shift: std_logic_vector(2 downto 0)) return integer is variable ret_var: integer range 0 to 7; begin case shift is when "000" => ret_var:=0; when "001" => ret_var:=1; when "010" => ret_var:=2; when "011" => ret_var:=3; when "100" => ret_var:=4; when "101" => ret_var:=5; when "110" => ret_var:=6; when "111" => ret_var:=7; when others => NULL; end case; return ret_var; end conv_int; --function to rotate input data to right by n function f_ror(din: std_logic_vector(7 downto 0); n: integer range 0 to 7) return std_logic_vector is variable ret_var: std_logic_vector(7 downto 0); begin ret_var:=din; for i in 0 to 7 loop if(i<(n)) then ret_var:=ret_var(0)&ret_var(7 downto 1); end if; end loop; return(ret_var); end f_ror; --function to rotate input data to left by n function f_rol(din: std_logic_vector(7 downto 0); n: integer range 0 to 7) return std_logic_vector is variable ret_var: std_logic_vector(7 downto 0); begin ret_var:=din; for i in 0 to 7 loop if(i<(n)) then ret_var:=ret_var(6 downto 0)&ret_var(7); end if; end loop; return(ret_var); end f_rol; --function to shift input data to right by n function f_shr(din: std_logic_vector(7 downto 0); n: integer range 0 to 7) return std_logic_vector is variable ret_var: std_logic_vector(7 downto 0); begin ret_var:=din; for i in 0 to 7 loop if(i<(n)) then ret_var:='0'&ret_var(7 downto 1); end if; end loop; return(ret_var); end f_shr; --function to shift input data to left by n function f_shl(din: std_logic_vector(7 downto 0); n: integer range 0 to 7) return std_logic_vector is variable ret_var: std_logic_vector(7 downto 0); begin ret_var:=din; for i in 0 to 7 loop if(i<(n)) then ret_var:=ret_var(6 downto 0)&'0'; end if; end loop; return(ret_var); end f_shl; signal n: integer range 0 to 7; --Architecture begins begin n<=conv_int(shift); process(din,n,con) begin case con is when "00" => dout <= f_ror(din,n); when "01" => dout <= f_rol(din,n); when "10" => dout <= f_shr(din,n); when "11" => dout <= f_shl(din,n); when others => NULL; end case; end process; end barrels_arch;

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_arith.all; use ieee.std_logic_unsigned.all; entity barrels_tb is end entity; architecture barrels_tb_arch of barrels_tb is component barrels is port ( din: in std_logic_vector(7 downto 0); shift: in std_logic_vector(2 downto 0); con: in std_logic_vector(1 downto 0); dout: out std_logic_vector(7 downto 0) ); end component; signal din: std_logic_vector(7 downto 0):="11111111"; signal shift: std_logic_vector(2 downto 0):="000"; signal con: std_logic_vector(1 downto 0):="00"; signal dout: std_logic_vector(7 downto 0); begin uut: barrels port map(din,shift,con,dout); process begin for b in 0 to 7 loop shift<=shift+1; wait for 20 ns; end loop; wait for 20 ns; for i in 0 to 2 loop con<=con+1; wait for 20 ns; end loop; end process; end;

Post Lab Subjective/Objective type Questions:

Upload Answer of following question before coming to next laboratory. Q1 How a procedure is written in VHDL? Explain by writing a procedure for shifting the 8 bit data by given number of digits. Q2 What are the differences in Functions and Procedures. Q3 Write a function in VHDL for converting integer number into binary (use b bit). Show how the function can be used in architecture. Q4 Which is correct expression for shift right of Din ( 4 bit data)? a. Din <= Din( 2 downto 0) &‟0‟ b. Din <= Din( 3 downto 1) &‟0‟ c. Din <=‟0‟ &Din( 2 downto 0) d. Din <= ‟0‟ & Din( 3 downto 1) Ans: a. Din <= Din( 2 downto 0) &‟0”

Conclusion: Thus, in this experiment we have implemented a Barrel Shift register by using functions and procedures in VHDL with various operations: Shift Right, Shift Left, Rotate Right, Rotate Left and Load.

Signature of faculty in-charge with Date: